# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Procurement of RIB and rubber boats with outboard engines and trailers for project I.D.Danube

Ref. number: HR-RS00034 – TD04

Lot 1: Two RIB boats with outboard engines and trailers

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

General Requirements

- All requirements stated and outlined in this document must be regarded as mandatory and the minimum acceptable criteria. All requirements outlined in this document are accompanied by the phrase "or equivalent".

- The tenderer must provide the specifications of the offered items in the Technical Offer (Column 3.- Specifications offered), including details such as the manufacturer, product type, model and accurate characteristics of offered equipment for requested values. Additionally, the tenderer must submit brochures and catalogue documentation indicating the requested parameters of the vehicles offered. All documentation must be provided in English or Serbian, both in hardcopy and electronic formats.

- If in the document ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER -Column 3 (Specifications offered), the tenderer only provide generic confirmations such as "yes", "compliant" or text copied directly from Column 2, the offer may not be considered, unless accompanied by supporting documentation (e.g., a product catalog or similar). Tenderers must provide comprehensive information to demonstrate that the offered product meets all required specifications. If the details provided in the offered specifications or supporting documentation differ from the required specifications listed in Column 2, the offer will be deemed non-compliant.

- Documentary evidence must be provided for each item listed in the document ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER where compliance with the relevant standard is claimed.

- The selected Contractor shall ensure that the offered boat bears a valid CE marking and comply with the applicable European Union legislation and standards. The tenderer shall submit, as part of the tender, the EU Declaration of Conformity (DoC) or an equivalent manufacturer's document demonstrating compliance with the applicable requirements. Upon delivery, the Contractor shall provide all documentation required for the registration of the vessel and the outboard engine in the Republic of Serbia, in accordance with the applicable national legislation.

- The selected contractor is required to provide all necessary documentation for the registration of the trailer in accordance with the relevant legal regulations in the Republic of Serbia. This includes the Certificate from the Agency for Traffic Safety confirming the trailer examination, as well as the technical inspection registration sheet, which is required for further trailer registration.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | RIB boat with outboard engine and trailer, Quantity: 2 Sets Required contents and characteristics of the Unit set: |  |  |  |
| 1.1 | Boat required technical characteristics: - Overall Length: min. 7.50m - Overall Beam (Width): min. 2.80m - Internal Length: min. 5.70m - Internal Width: min. 1.50m - Tube Diameter: min. 54cm - Number of Chambers: min. 6 - Aluminium hull and hypalon tubes - Fuel Tank Capacity: min. 280L Engine required technical characteristics: - Power: min. 180 kW - Engine Type: 4-stroke, petrol outboard - Number of Cylinders: 6 - Displacement: min. 3,500 cc, max 3,600 cc - Fuel Delivery: Programmed Fuel Injection (PGM-FI) - Transom Height (X-Type): min. 635 mm - Gear Ratio: 2.00:1 - Alternator: min. 90-Amp, 60A charging capacity - Steering: Remote control Engine support system - Hydraulic system for vertical engine height adjustment, compatible with standard outboard mounting systems, intended for transom installation. Outboard motor trolley for assisting boat electric outboard motor. - Steel trolley with large wheels suitable for transporting small electric outboard motor. Carrying capacity of 50kg. Engine accessories - Outboard engine flushing muffs suitable for offered outboard engine. Spare propeller - Suitable inox propeller for the engine. Complete installation and rigging: - Central console with seating - Integrated electrical wiring system - Switch panel with circuit protection Battery system: - Quantity: 2 units - Type: AGM deep cycle - Capacity: 90 Ah each or bigger Hydraulic steering system: - Fully hydraulic steering system with steering wheel - Corrosion-resistant components Boat Additional Equipment: Electric searchlight: - Control: console-mounted joystick or wireless remote control - Rotation: 360° horizontal - Power supply: 12V system Night vision navigation system: - Type: marine night vision camera - Mount: fixed front position - Image resolution: minimum 1280 x 1024 - Compatible with MFD Marine VHF radio: - Class D VHF DSC marine radio with integrated GPS receiver, - Dual-channel AIS receiver, - NMEA 2000 and NMEA 0183 network support, - IPX7 waterproof housing, - Ability to mount the hand microphone on either the front or rear side, large backlit display for day and night operation, - MOB (Man Overboard) function, - ATIS compatibility, and integration capability with Lowrance chartplotters. AIS transponder: - AIS Class B transponder, supporting NMEA 2000 and NMEA 0183 standards, enabling automatic transmission and reception of AIS data for vessel identification and tracking, with low power consumption, - IPX7 protection rating, fixed installation, - LED status indicators, and compatibility with onboard navigation systems, supplied with GPS antenna, connection cables, and mounting accessories. Multifunction display with chartplotter and sonar: - Chartplotter with high-definition SolarMAX display and multi-touch interface, - Integrated sonar functionality, support for autopilot and radar integration, NMEA 2000 and Ethernet connectivity, supplied with dual transducer system including combined CHIRP, - SideScan and DownScan transducer and through-hull CHIRP transducer (600 W), providing depth and temperature data, with low-profile installation. Mooring and anchoring package: - Fenders, mooring lines, anchor with anchor line or chain, oar, and buoy, intended for safe berthing and anchoring operations. Firefighting mount: - Fixed bracket for nozzle, stainless steel with quick release mechanism. Universal water nozzle (full-spray jet) with a diameter of 52 mm and compatible with a 'C' fire hose. Hull branding: - Identification sticker with text " ВАТРОГАСЦИ-СПАСИОЦИ 193", letter height approx. 15cm , suitable for application on Hypalon. - Custom placement. Trailer required technical characteristics: - Twin-axle braked boat trailer with minimum payload capacity of 2,500 kg, suitable for the offered boat, engine and installed equipment, designed for the transport of boats up to 8 m in length. Assisting Boat: Inflatable rescue boat: - Inflatable whitewater raft with an overall length of 4.25 m and beam of 2.14 m, designed with high-load-capacity inflatable tubes, suitable for up to 8+1 persons, with a maximum load capacity of 1,000 kg, intended for sport, competition and recreational rafting use. -Oar set, minimum 6, aluminium shaft with plastic blades Electric outboard motor: - Portable 1 kW electric outboard motor with integrated lithium battery, suitable for tenders, dinghies and small boats, featuring quiet operation, zero-emission propulsion and hydrogeneration capability. Floating platform: - Floating platform of hypalon-neoprene dropstitch, intended to provide operational support to a workboat, with a stable surface suitable for personnel access, equipment handling and service activities. Dimensions: approximately 5 m x 1.5 m. |  |  |  |
| 1.2 | Floating pump, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Pumping Performance: - Floating high-pressure pump with capacity of approximately 1200 l/min, powered by a petrol engine (minimum 2 kW), self-priming design, maximum operating pressure of approximately 3 bar, standard connection for firehose, lightweight construction for easy transport, and housing suitable for operation in shallow and debris-laden water. |  |  |  |
| 1.3 | Lifeguard equipment, Quantity: 6 pieces |  |  |  |
|  | Dry rescue suits: Drysuit designed for professional water operations, constructed from durable waterproof and breathable materials, with sealed seams, integrated waterproof zipper, reinforced high-wear areas, adjustable seals for wrists and neck, and ergonomic fit to ensure thermal protection and mobility in cold-water conditions. Rescue life jackets: Personal flotation device designed for rescue operations, with high buoyancy, adjustable wrap-around design for secure fit, multiple attachment points for rescue equipment, high-visibility elements, and durable construction suitable for professional water use. |  |  |  |
| 1.4 | Vests for endangered persons, Quantity: 10 pieces |  |  |  |
|  | Additional life jackets: Personal flotation device compliant with EN 12402-3 or EN 12402-4 standards, designed to provide adequate buoyancy and safety for water operations, with adjustable fit and durable construction suitable for marine use. |  |  |  |
| 1.5 | Rescue buoys, Quantity: 2 pieces |  |  |  |
|  | Required technical characteristics: Rescue buoy designed for water rescue operations, constructed from durable, buoyant materials, with high-visibility finish and integrated line or attachment points for effective deployment and retrieval. |  |  |  |
| 1.6 | Rescue hooks/poles, Quantity: 2 pieces |  |  |  |
|  | Required technical characteristics: Boat hook designed for mooring and retrieval operations, constructed from corrosion-resistant materials, with extended reach and non-slip handle for safe handling in marine environments. |  |  |  |
| 1.7 | Rescue stretcher/basket, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Rescue stretcher designed for water operations, constructed from durable and buoyant materials, with integrated securing straps and lifting points for safe transport and evacuation of injured persons. |  |  |  |
| 1.8 | Spinal board, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Spinal board designed for immobilization and transport of injured persons, constructed from durable and buoyant materials, with integrated handholds and securing points, compatible with standard restraint systems for safe rescue operations. |  |  |  |
| 1.9 | First aid kits, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: First aid kit equipped with essential medical supplies for emergency response, stored in a durable and portable case, suitable for marine and rescue operations. |  |  |  |
| 1.10 | Disposable protective kits, Quantity: 20 pieces |  |  |  |
|  | Required technical characteristics: Body recovery bag designed for safe handling and transport, constructed from durable, leak-resistant materials with secure closure system and reinforced carrying handles. |  |  |  |
| 1.11 | Waterproof and floating radio stations, Quantity: 6 pieces |  |  |  |
|  | Required technical characteristics: Waterproof handheld VHF radio station with floating design and high-visibility housing, transmission power up to 6 W, suitable for reliable marine communication in emergency and operational conditions. |  |  |  |
| 1.12 | Floating rescue rope, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Floating rope made of buoyant synthetic material, diameter 10 mm and length 200 m, suitable for marine and rescue operations. |  |  |  |
| 1.13 | Marine binoculars, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Binoculars with 10× magnification and 50 mm objective lenses, providing clear and stable viewing, with durable construction suitable for outdoor and marine use, dimensions approximately 195 × 178 × 68 mm and weight approximately 1020 g. |  |  |  |
| 1.14 | Portable flashlights, Quantity: 6 pieces |  |  |  |
|  | Required technical characteristics: Handheld LED flashlight with high-intensity output up to approximately 5000 lumens and beam distance of approximately 500 m, powered by rechargeable Li-ion battery with USB-C charging, multiple lighting modes including turbo and strobe, impact-resistant aluminium housing, IP68 waterproof rating (submersible up to 2 m), and rear tactical switch for rapid operation, suitable for rescue and field use. |  |  |  |
| 1.15 | Battery-powered chainsaw, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Cordless chainsaw powered by a rechargeable battery system, supplied with battery (approx. 12.0 Ah), charger, and carrying case, suitable for cutting applications in field, maintenance, and service operations, with guide bar length of approx. 40 cm. |  |  |  |
| 1.16 | Battery-powered angle grinder, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Cordless angle grinder powered by rechargeable battery system, supplied with battery (approx. 5.0 Ah), charger, and carrying case, suitable for cutting and grinding applications in field and maintenance operations. |  |  |  |
| 1.17 | Battery-powered reciprocating saw, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Cordless reciprocating saw powered by rechargeable battery system, supplied with battery (approx. 5.0 Ah), charger, and carrying case, suitable for cutting applications in field and maintenance operations. |  |  |  |
| 1.18 | Portable floodlight with tripod, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Portable work light with tripod, powered by rechargeable battery system, luminous flux minimum 4,500 lm, multiple light sources with adjustable output levels, supplied with battery and charger, suitable for illumination in field and maintenance operations. |  |  |  |
| 1.19 | Extension cable, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Cable reel with approximately 50 m of cable (3 × 1.5 mm²), equipped with four power outlets, suitable for portable power distribution in field and maintenance operations. |  |  |  |
| 1.20 | Portable inverter generator, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Inverter generator powered by petrol engine, output power approximately 5.5 kW to 7 kW, low-noise operation, equipped with electric start, suitable for stable power supply in field and emergency applications. |  |  |  |
| 1.21 | Portable power station, Quantity: 1 piece |  |  |  |
|  | Required technical characteristics: Portable power station with capacity of approximately 2048 Wh and continuous output up to 3000 W, based on LFP battery technology with extended lifecycle, equipped with multiple AC and DC outputs, fast charging capability, integrated high-efficiency inverter, and durable housing suitable for field and rescue use. |  |  |  |

| All items | Delivery of all items to following address: Srete Glavačkog 30 , 21220, Bečej, Republic of Serbia. |  |  |  |
| --- | --- | --- | --- | --- |
| All items | Installation and testing of all functions of the instrument, to manufacturers standard on samples commonly used for the corresponding instrument. Installed equipment must be tested as system, compatible with existing system. |  |  |  |
| All items | Training one day event/training for minimum 5 Contracting Authority employees. |  |  |  |
| All items | LABELING OF THE VEHICLE: It is necessary to attach the label of the Sector for Emergency Management on the driver and co-driver doors. Below the label of the Sector for Emergency Management emergency number 193 (in white). On the hood panel of the vehicle, place the Cyrillic inscription " ВАТРОГАСЦИ- СПАСИОЦИ " in white. Vehicle shall also be visibly labelled as per EC visibility requirements (according to the latest Communication and Visibility Requirements for EU-funded external action, laid down and published by the European Commission website). Each vehicle must be marked with durable/UV resistant plastic stickers with above mentioned content. The bidder shall produce a draft layout for written approval by customer prior to production. |  |  |  |
| All items | The warranty period should be minimum 2 years, regardless of the distance traveled. During the warranty period, any malfunction of the products covered by the warranty will be solved free of charge by the supplier within 30 days from the date of the notification of the fault to the service unit of the products, without additional costs to the contracting authority. The tenderer must provide a list of authorized service centers for vehicles in the territory of the Republic of Serbia. Furthermore, the tenderer is obligated to provide a list of authorized services for superstructure and equipment within the territory of the Republic of Serbia. If the tenderer does not have its own authorized service center, it is necessary to provide a contract(s) with an authorized service center (concluded at least 6 months before the announcement of this tender). Authorized service centers should provide 24/7 mobile support service. The tenderer is also required to supply manuals and other necessary documentation for the installation, operation, maintenance, and repair of the vehicles and equipment. These manuals should be provided in both English and Serbian languages, in both hardcopy and electronic formats. |  |  |  |

<Signature of authorised representative of the legal entity >

< Name and position of authorised representative of the legal entity >

< Date >